DRONE TECHNOLOGY AND ITS TRANSFORMATIVE APPLICATIONS

PROJECT WORK

SUBMITTED BY:

RENUBALA SHADANGI  23BEEN0025

KUMKUM SHADANGI    23BEEN0024

KULDEEP JANGID            23BEEN0029

AIM: Thrust estimation of a propeller system using aerodynamic load modelling.

PROBLEM STATEMENT:

Model a propeller as a simplified aerodynamic load driven by a motor in Amesim. Simulate the relationship between rotational speed(RPM) and thrust generation. Analyze how motor speed affects upward force, essential for stable drone lift design.

PROPOSED SIMULATION APPROACH IN AMESIM:

1.System architecture:

Model will consist of

Signal and control library for generating RPM setpoints and PWM signals.

Electro mechanical library for modelling the DC or BLDC motor.

Mechanical library for rotational inertia and shaft connections.

Aerospace library for propeller thrust as aerodynamic load.

2.Component selection and modelling

Use a DC or BLDC motor.

Input: voltage or pwm duty cycle.

Output :Shaft torque and RPM

Use the propeller component.

Aerodynamic torque: T=k.w2

Thrust force: Ft=CT*p*n2*D4

Where CT=thrust coefficient

P=air density

n=rotational speed

D=propeller diameter

Connect motor shaft to propeller load via a rotational inertia element.

Use a force sensor to compute thrust from w using thrust -RPM equation.

3.Thrust -RPM relationship derivation

Convert n to w(rad/s)

Let Kthrust=CTpD4/(4*3.142)

Thus Ft=Kthrust*w2

JUSTIFICATION OF SUBSYSTEMS/BLOCKS USED:

1.Voltage source/PWM generator:

Provides controlled electrical input to the motor

Allows simulation of different operating conditions(step ramp, sinusoidal points)

Enables study of motor response.

2.DC/BLDC motor component:

Physically accurate representation.

Native integration, parameter flexibility.

3.Shaft/mechanical connection:

Provides energy transfer, maintain angular velocity continuity.

4.Rotational inertia:

Critical for dynamics, affects acceleration time.

5.Propeller component:

Built in aerodynamics ,altitude effects, advanced ratio effects

6.Rotational speed sensor:

Essential for rpm-thrust analysis, measures actual shaft speed.

7.Torque sensor:

Measures motor output torque, verifies power balance.

CONCLUSION AND INFERENCE:

1.Thrust-RPM relationship confirmation:

Thrust exhibits a quadratic relationship with RPM.

This confirms fundamental aerodynamic theory for propellers in axial flow.

2.Motor-propeller matching:

Above 4000 RPM, efficiency drops due to increased drag losses.

3.Performance limitations:

Continuous operation at maximum thrust unsustainable. Peak efficiency at 70-80%.

Safe continuous operation :0-4000RPM

4.Response: First order dominant with 100-200ms time constant.

Minimize rotating inertia, maximize torque capability.

5.The simulation provides a validated framework of propeller – motor system design.